# Offline, Privacy-Preserving Hindi Voice Assistant on Raspberry Pi

## 1. Methodology

### 1.1 System Design

The system is a fully offline, privacy-preserving Hindi voice assistant running entirely on a Raspberry Pi 4 CPU with no external APIs or cloud dependencies.

Major Components:

- Speech-to-Text (ASR): Converts Hindi speech into text using an ONNX-optimized Wav2Vec2 model.

- Intent Recognition: Lightweight keyword-based intent detection using rapidfuzz.

- Action Handling: Modular Python functions execute responses for detected intents.

- Text-to-Speech (TTS): Offline Hindi speech synthesis via espeak-ng.

The assistant continuously listens to the microphone, processes speech locally, identifies user intent, executes the relevant action, and responds with Hindi TTS, ensuring real-time performance and user privacy.

### 1.2 Speech-to-Text (ASR)

- Built on fine-tuned Wav2Vec2 Hindi model, converted to ONNX for optimized CPU inference.

- ONNX Runtime executes efficiently on ARM CPUs.

- Raspberry Pi Optimizations:

  - OPENBLAS_CORETYPE = ARMV8

  - Graph Optimization Level = ORT_ENABLE_ALL

  - CPU-only execution with intra_op_num_threads = 2

- Audio Processing:

  - 16 kHz sampling rate

  - Energy-based silence detection

  - RMS thresholding to reduce unnecessary computation

Benefits:

- Zero internet dependency

- Low-latency inference

- Full data privacy

- Edge-device AI processing

### 1.3 Intent Recognition

- Uses intent.json for keyword mapping.

- Fuzzy matching via rapidfuzz.partial_ratio().

- Match threshold = 70% for intent selection.

- Supported intents:

  - WORD_DEFINITION

  - CALCULATION

  - CONVERSION_QUERY

  - GK_QUERY

  - FACTS_QUERY

  - HISTORY_QUERY

  - TOURISM_QUERY

  - EMERGENCY_QUERY

  - WEBSITE_QUERY

  - GREETING

Advantages:

- Computationally lightweight

- Quick execution on Raspberry Pi

- Easily expandable by adding new keywords

### 1.4 Action Handling

- Implemented in actions.py.

- Function handle_intent(intent, text) routes each intent to its corresponding function:

  - word_definition(), calculation_query(), conversion_query(), gk_query(), facts_query(), history_query(), tourism_query(), emergency_query(), website_query(), greeting()

- Modular design ensures:

  - Clear separation of concerns

  - Offline execution

  - Easy scalability

### 1.5 Text-to-Speech (TTS)

- Offline synthesis using espeak-ng with Hindi voice (-v hi) via stdin piping.

- Multithreaded execution prevents blocking microphone input.

- Advantages:

  - Lightweight and fast

  - Fully offline

  - Efficient CPU utilization

## 2. System Architecture

Pipeline Flow:

Microphone Input
      ↓
Energy-Based Voice Detection
      ↓
Wav2Vec2 ONNX ASR
      ↓
Decoded Hindi Text
      ↓
Fuzzy Intent Detection
      ↓
Action Execution
      ↓
eSpeak NG TTS
      ↓
Speaker Output

Key Features:

- Multithreaded TTS (is_speaking flag prevents self-triggering)

- CPU-only ONNX inference

- Graph-level optimization

- Energy-efficient recording

- Modular and scalable design

## 3. Hardware Utilization

- Platform: Raspberry Pi 4 (ARM Cortex-A72, 4GB RAM)

- CPU Optimization: 2 intra-op threads, CPU-only execution

- Memory Management: ASR model loaded once; temporary audio buffers reused

- Audio Hardware: USB microphone; 3.5mm audio or HDMI output

Performance: Stable operation with real-time responsiveness and no CPU throttling under moderate use.

## 4. Optimization Techniques

- ONNX Conversion: PyTorch → ONNX reduced inference time

- Graph-Level Optimization: ORT_ENABLE_ALL improves computation efficiency

- Thread Optimization: 2 threads prevent CPU overload

- Energy-Based Recording: Stops after silence, reducing unnecessary ASR computation

- Multithreaded TTS: Enhances responsiveness

- Lightweight Intent Matching: Fuzzy keyword approach decreases latency

## 5. Challenges

Hindi ASR:

- Accent variability

- Code-mixed Hindi-English speech

- Background noise sensitivity

- Word boundary ambiguities

Hindi TTS:

- Robotic voice quality

- Pronunciation inconsistencies

- Limited prosody and dialect handling

## 6. Results

- Fully offline Hindi speech processing

- Runs entirely on Raspberry Pi CPU

- Latency < 2 seconds

- No cloud dependency

- Full data privacy

- Modular intent expansion supported

## 7. Conclusion

This project demonstrates a real-time, privacy-preserving Hindi voice assistant operating completely offline on Raspberry Pi.

Key Features:

- ONNX-optimized Wav2Vec2 ASR

- Lightweight fuzzy intent recognition

- Modular Python action handling

- Offline Hindi TTS with espeak-ng

Applications:

- Rural or low-connectivity regions

- Privacy-sensitive environments

- Educational and embedded AI research

Future Enhancements:

- Neural Hindi TTS models

- Wake-word detection

- Model quantization

- Expanded offline knowledge base

- Integration with hardware-based home automation